Norwich City Council Inflatables Inspection Checklist

Date:						Inspected by:

Type/description of equipment:

Location:

Person seen & position:

Operator name:

Operator address:

Tel:						Email:

Structure relying on a continuous supply of air to maintain its shape, or on which users may play, bounce or slide.

| Inspection Criteria | Yes | No | Comments |
| --- | --- | --- | --- |
| Anchorage |  |  |  |  |
| Minimum 6 anchor points |  |  |  |
| Evenly distributed |  |  |  |
| D/O/Triangle ring welded metal |  |  |  |
| Ground pegs angled 30-45 degrees |  |  |  |
| Peg length minimum 380mm & 16mm diameter |  |  |  |
| Pegs free from cracks / sharp edges |  |  |  |
| Ground pegs / ballast protected |  |  |  |
| Ballast 163kg / 163 liters |  |  |  |
| High level anchor points used |  |  |  |
| Fiber rope (no bungee cord) |  |  |  |
| Rope diameter 18mm-45mm |  |  |  |
| Tunnels / squeezes / mesh |  |  |  |  |
| Tunnel 75cm-2m length: minimum 50cm diameter |  |  |  |
| Tunnel more than 2m length: minimum 75cm diameter |  |  |  |
| Squeeze length: maximum 75cm |  |  |  |
| Squeeze opening: minimum 40cm |  |  |  |
| Mesh: maximum size 8mm |  |  |  |
| Slide containment |  |  |  |  |
| Platform height > 0.6m: walls required |  |  |  |
| Minimum wall / roof height 0.75m |  |  |  |
| Platform height 0.6m-3m: wall height equal to user (1.8m ok for all) |  |  |  |
| Platform height 3m-6m: wall height minimum 1.25x height of user / permanently roofed |  |  |  |
| Platform height > 6m: permanently roofed |  |  |  |
| Positioning |  |  |  |  |
| Safety matting at entrance |  |  |  |
| No adjacent hard surfaces |  |  |  |
| Ceiling clearance (if used indoors) |  |  |  |
| Internal pressure |  |  |  |  |
| Manometer available & regularly checked |  |  |  |
| Minimum 100mm water gauge / 10millibar |  |  |  |
| No obvious seam deterioration or rips |  |  |  |
| No signs of over-pressurisation |  |  |  |
| Blower rated IP23B (EN 60529) |  |  |  |
| Mesh guard covering blade |  |  |  |
| Blade free of debris |  |  |  |
| Blade not damaged / worn / missing |  |  |  |
| Hinged flap to blower outlet |  |  |  |
| Blower positioned minimum 1.2m from walled side & 2.5m from open side |  |  |  |
| Type / size of blower appropriate |  |  |  |
| Safe operation |  |  |  |  |
| Manufacturers operating instructions available / dry hire instructions |  |  |  |
| Max number of users & height marked on equipment and followed |  |  |  |
| Bigger users separated from smaller users |  |  |  |
| Policy - no shoes/ glasses / intoxicated / climbing / hanging / empty pockets |  |  |  |
| Anchor points regularly checked |  |  |  |
| Anemometer available & regularly used |  |  |  |
| Max wind loading known |  |  |  |
| Formal inspection |  |  |  |  |
| PIPA - tag attached to inflatable (tag number & blower ID match certificate) |  |  |  |
| ADIPS - checked DOC (Declaration Operational Compliance Certificate) from device ID number |  |  |  |